UGC PROJECT REPORT

On

FAST FRACTAL COMPRESSION USING VARIANCE ORDERED BLOCK SEARCH ALGORITHM

Submitted By-

MOHAMMED

In partial fulfilment of requirement

for the fellowship of UGC

in

ELECTRONICS & COMMUNICATION ENGINEERING

SCHOOL OF ENGINEERING                               UNIVERSITY GRANTS COMMISSION

COCHIN UNIVERSITY OF SCIENCE AND TECHNOLOGY

KOCHI – 682022

COCHIN UNIVERSITY OF SCIENCE AND TECHNOLOGY

DIVISION OF ELECTRONICS ENGINEERING

SCHOOL OF ENGINEERING 		UNIVERSITY GRANTS COMMISSION

CERTIFICATE

Certified that this report titled

FAST FRACTAL COMPRESSION USING VARIANCE ORDERED BLOCK SEARCH ALGORITHM

is a bonafide record of UGC project work of

MOHAMMED

towards the partial fulfilment of the requirement for the award of fellowship of UGC in Electronics and Communication Engineering  of Cochin University of Science & Technology.

Project-Guide									Head

Mrs. Deepa Sankar								    Dr. R .Gopikakumari

ACKNOWLEDGEMENT

I express my heartfelt thanks and gratitude to my parents, without whose encouragement, unwavering support and blessings, successful completion of the project would have been a distant dream. I express my genuine gratitude to my project guide Mrs. Deepa Sankar and Head of Department Dr.R.Gopikakumari for providing me the required support and encouragement for the successful completion of the project in the given time frame. Last but not the least I would like to thank all the teachers and our classmates who helped in one way or other helped my project come alive.

I above all thank Almighty God for having given me such a wonderful project to work on, under an equally wonderful guide.

MOHAMMED

ABSTRACT

A new fast fractal encoding algorithm based on the variances of image blocks is proposed. With the domain blocks sorted according to their variances, and the best matched domain block to a given range block is searched in the order that the variance-distance is closer. A great number of domain blocks could be safely rejected by the prior comparison of   the current minimum distortion and variance difference between the candidate domain block and the range block during the search process. It was proved that our algorithm produces completely identical fractal codes with that of the conventional full search in reduced time. The simulation results confirmed the effectiveness of the proposed algorithm.

# Fractal Image Compression

CHAPTER 1:  INTRODUCTION

1. Introduction

With the advance of the information age the need for mass information storage and fast communication links grows. Storing images in less memory leads to a direct reduction in storage cost and faster data transmissions. These facts justify the efforts, of private companies and universities, on new image compression algorithms.

Storing an image on a computer requires a very large memory. This  problem  can  be  averted  by  the  use  of  various  image  compression  techniques. Most  images contain  some  amount  of  redundancy  that  can  be  removed  when  the  image  is  stored  and  then  replaced  when  it  is  reconstructed.

Fractal  image  compression  is  a  recent  technique  based  on  the  representation  of  an  image. The  self-transformability  property  of  an  image  is  assumed  and  exploited  in  fractal  coding. It provides high compression ratios and fast decoding. Apart  from  this  it  is  also  simple  and  is  an  easily  executable  technique.

Images are stored on computers as collections of bits (a bit is a binary unit of information which can answer “yes” or “no” questions) representing pixels or points forming the picture elements. Since the human eye can process large amounts of information (some 8 million bits), many pixels are required to store moderate quality images. These bits provide the “yes” and “no” answers to the 8 million questions that determine the image.

Most data contains some amount of redundancy, which can sometimes be removed for storage and replaced for recovery, but this redundancy does not lead to high compression ratios. An image can be changed in many ways that are either not detectable by the human eye or do not contribute to the degradation of the image.

The standard methods of image compression come in several varieties. The current most popular method relies on eliminating high frequency components of the signal by storing only the low frequency components (Discrete Cosine Transform Algorithm). This method is used on JPEG (still images), MPEG (motion video images), H.261 (Video Telephony on ISDN lines), and H.263 (Video Telephony on PSTN lines) compression algorithms.

Fractal Compression was first promoted by M.Barnsley, who founded a company based on fractal image compression technology but who has not released details of his scheme. The first public scheme was due to E.Jacobs and R.Boss of the Naval Ocean Systems Centre in San Diego who used regular partitioning and classification of curve segments in order to compress random fractal curves (such as political boundaries) in two dimensions. A doctoral student of Barnsley’s, A. Jacquin, was the first to publish a similar fractal image compression scheme.

1.1 A Brief History of Fractal Image Compression

The birth of fractal geometry (or rebirth, rather) is usually traced to IBM mathematician Benoit B. Mandelbrot and the 1977 publication of his seminal book “The Fractal Geometry of Nature”. The book put forth a powerful thesis: traditional geometry with its straight lines and smooth surfaces does not resemble the geometry of trees and clouds and mountains. Fractal geometry, with its convoluted coastlines and detail ad infinitum, does.

This insight opened vast possibilities. Computer scientists, for one, found a mathematics capable of generating artificial and yet realistic looking landscapes, and the trees that sprout from the soil. And mathematicians had at their disposal a new world of geometric entities.

It was not long before mathematicians asked if there was a unity among this diversity. There is, as John Hutchinson demonstrated in 1981, it is the branch of mathematics now known as Iterated Function Theory. Later in the decade Michael Barnsley, a leading researcher from Georgia Tech, wrote the popular book “Fractals Everywhere”. The book presents the mathematics of Iterated Functions Systems (IFS), and proves a result known as the Collage Theorem. The Collage Theorem states what an Iterated Function System must be like in order to represent an image.

This presented an intriguing possibility. If, in the forward direction, fractal mathematics is good for generating natural looking images, then, in the reverse direction, could it not serve to compress images? Going from a given image to an Iterated Function System that can generate the original (or at least closely resemble it), is known as the inverse problem. This problem remains unsolved.

Barnsley, however, armed with his Collage Theorem, thought he had it solved. He applied for and was granted a software patent and left academia and found Iterated Systems Incorporated (US patent 4,941,193. Alan Sloan is the co-grantee of the patent and co-founder of Iterated Systems.) Barnsley announced his success to the world in the January 1988 issue of BYTE magazine. This article did not address the inverse problem but it did exhibit several images purportedly compressed in excess of 10,000:1. Alas, it was a slight of hand. The images were given suggestive names such as "Black Forest" and "Monterey Coast" and "Bolivian Girl" but they were all manually constructed. Barnsley's patent has come to be derisively referred to as the "graduate student algorithm."

Attempts to automate this process have continued to this day, but the situation remains bleak. As Barnsley admitted in 1988: "Complex color images require about 100 hours each to encode and 30 minutes to decode on the Masscomp [dual processor workstation]." That's 100 hours with a person guiding the process.

Ironically, it was one of Barnsley's PhD students that made the graduate student algorithm obsolete. In March 1988, according to Barnsley, he arrived at a modified scheme for representing images called Partitioned Iterated Function Systems (PIFS). Barnsley applied for and was granted a second patent on an algorithm that can automatically convert an image into a Partitioned Iterated Function System, compressing the image in the process. (US patent 5,065,447. Granted on Nov. 12 1991.) For his PhD thesis, Arnaud Jacquin implemented the algorithm in software, a description of which appears in his landmark paper "Image Coding Based on a Fractal Theory of Iterated Contractive Image Transformations." The algorithm was not sophisticated, and not speedy, but it was fully automatic. This came at price: gone was the promise of 10,000:1 compression. A 24-bit color image could typically be compressed from 8:1 to 50:1 while still looking "pretty good." Nonetheless, all contemporary fractal image compression programs are based upon Jacquin's paper.

That is not to say there are many fractal compression programs available. There are not. Iterated Systems sell the only commercial compressor/decompressor, an MS-Windows program called "Images Incorporated." There are also an increasing number of academic programs being made freely available. Unfortunately, these programs are of merely academic quality.

This scarcity has much to do with Iterated Systems tight lipped policy about their compression technology. They do, however, sell a Windows DLL for programming. In conjunction with independent development by researchers elsewhere, therefore, fractal compression will gradually become more pervasive. Whether it becomes all-pervasive remains to be seen.

# CHAPTER 2: WHAT IS FRACTAL IMAGE COMPRESSION?

Imagine a special type of photocopying machine that reduces the image to be copied by half and reproduces it three times on the copy (see Figure 1). What happens when we feed the output of this machine back as input? Figure 2 shows several iterations of this process on several input images. We can observe that all the copies seem to converge to the same final image, the one in 2(c). Since the copying machine reduces the input image, any initial image placed on the copying machine will be reduced to a point as we repeatedly run the machine; in fact, it is only the position and the orientation of the copies that determines what the final image looks like.

The way the input image is transformed determines the final result when running the copy machine in a feedback loop. However we must constrain these transformations, with the limitation that the transformations must be contractive (see contractive box), that is, a given transformation applied to any two points in the input image must bring them closer in the copy. This technical condition is quite logical, since if points in the copy were spread out the final image would have to be of infinite size. Except for this condition the transformation can have any form.In practice, choosing transformations of the form is sufficient to generate interesting transformations called affine transformations of the plane. Each can skew, stretch, rotate, scale and translate an input image.

wixy=aibicidixy+eifi

A common feature of these transformations that run in a loop back mode is that for a given initial image each image is formed from a transformed (and reduced) copies of itself, and hence it must have detail at every scale. That is, the images are fractals. This method of generating fractals is due to John Hutchinson.

Barnsley suggested that perhaps storing images as collections of transformations could lead to image compression. His argument went as follows: the image in Figure 3 looks complicated yet it is generated from only 4 affine transformations.

Each transformation wi is defined by 6 numbers, ai, bi, ci, di, ei, and fi , see eq(1), which do not require much memory to store on a computer (4 transformations x 6 numbers /transformations x 32 bits /number = 768 bits). Storing the image as a collection of pixels, however required much more memory (at least 65,536 bits for the resolution shown in Figure 2). So if we wish to store a picture of a fern, then we can do it by storing the numbers that define the affine transformations and simply generate the fern whenever we want to see it. Now suppose that we were given any arbitrary image, say a face. If a small number of affine transformations could generate that face, then it too could be stored compactly. The trick is finding those numbers.

2.1 Why The Name “Fractal”

The image compression scheme described later can be said to be fractal in several senses. The scheme will encode an image as a collection of transforms that are very similar to the copy machine metaphor. Just as the fern has detail at every scale, so does the image reconstructed from the transforms. The decoded image has no natural size; it can be decoded at any size. The extra detail needed for decoding at larger sizes is generated automatically by the encoding transforms. One may wonder if this detail is “real”; we could decode an image of a person increasing the size with each iteration, and eventually see skin cells or perhaps atoms. The answer is, of course, no. The detail is not at all related to the actual detail present when the image was digitized; it is just the product of the encoding transforms which originally only encoded the large-scale features. However, in some cases the detail is realistic at low magnifications, and this can be useful in Security and Medical Imaging applications. Figure 4 shows a detail from a fractal encoding of “Lena” along with a magnification of the original.

2.2 Properties of Fractals

A  set  F  is  said  to  be  a  fractal  if  it  possesses  the  following  properties.

- F is found to contain detail at every scale.

- F is self-similar.

- The  fractal  dimension  of  F  is  greater  than  it’s topological  dimension.

- F has got a simple algorithmic description.

2.3 Affine Transformations

An affine transformation is any transformation that preserves collinearity (i.e., all points lying on a line initially still lie on a line after transformation) and ratios of distances (e.g., the midpoint of a line segment remains the midpoint after transformation). In this sense, affine indicates a special class of projective transformations that do not move any objects from the affine space R3 to the plane at infinity or conversely. An affine transformation is also called an affinity.

Geometric contraction, expansion, dilation, reflection, rotation, shear, similarity transformations, spiral similarities, and translation are all affine transformations, as are their combinations. In general, an affine transformation is a composition of rotations, translations, dilations, and shears.

While an affine transformation preserves proportions on lines, it does not necessarily preserve angles or lengths. Any triangle can be transformed into any other by an affine transformation, so all triangles are affine and, in this sense, affine is a generalization of congruent and similar.

These  are  combinations  of  rotation, scaling  and  translation  of  the  co-ordinate  axis  in  an  N-dimensional  space. The  figure  shows  an  example  of  an  affine  transformation  W  which  moves  towards W(f)-that  is  it  is  a  contractive  transformation.

2.4 How much Compression can Fractal achieve?

The compression ratio for the fractal scheme is hard to measure since the image can be decoded at any scale. For example, the decoded image in Figure 3 is a portion of a 5.7 to 1 compression of the whole Lena image. It is decoded at 4 times its original size, so the full decoded image contains 16 times as many pixels and hence this compression ratio is 91.2 to 1. This many seem like cheating, but since the 4-times-later image has detail at every scale, it really is not.

CHAPTER 3: ENCODING IMAGES

The theorem tells us that transformation W will have a unique fixed point in the space of all images. That is, whatever image (or set) we start with, we can repeatedly apply W to it and we will converge to a fixed image.

Suppose we are given an image f that we wish to encode. This means we want to find a collection of transformations w1, w2, ...,wN and want f to be the fixed point of the map W. In other words, we want to partition f into pieces to which we apply the transformations wi , and get back the original image f .

A typical image of a face does not contain the type of self-similarity like the fern. The image does contain other type of self-similarity. Figure 5 shows regions of Lena identical, and a portion of the reflection of the hat in the mirror is similar to the original. These distinctions form the kind of self-similarity; rather than having the image be formed by whole copies of the original (under appropriate affine transformations), here the image will be formed by copies of properly transformed parts of the original. These transformed parts do not fit together, in general, to form an exact copy of the original image, and so we must allow some error in our representation of an image as a set of transformations.

3.1 Proposed Algorithm

Encoding:

The following example suggests how the Fractal Encoding can be done. Suppose that we are dealing with a 128 x 128 image in which each pixel can be one of 256 levels of gray. We called this picture Range Image. We then reduce by averaging (down sampling and lowpass-filtering) the original image to 64 x 64. We called this new image Domain Image. We then partitioned both images into blocks 4 x 4 pixels (see Figure 6).

We performed the following affine transformation to each block:

Di,j=αDi,j+t0

where α=0,1, α∈R and t0∈-255,255, t0∈Z

In this case we are trying to find linear transformations of our Domain Block to arrive to the best approximation of a given Range Block. Each Domain Block is transformed and then compared to each Range Block Rk,l . The exact transformation on each domain block, i.e. the determination of a and to is found minimizing

minm.n(Rk,l)m.n-(ΓDi,j)m,n

with respect to α and t0

α=Ns2m,nDi,jm,nDk,lm,n-m,nDi,jm,nm,nRk,lm,nNs2m,n(Di,jm,n2-m,nDi,jm,n2

t0=(m,n(Di,j)m,n)2-m,n(Rk,l)m,n)2Ns2m,n((Di,j)m,n)2-(m,n(Di,j)m,n)2

where m, n, Ns = 2 or 4 (size of blocks).

Each transformed domain block G(Di,j) is compared to each range block Rk,l in order to find the closest domain block to each range block. This comparison is performed using the following distortion measure.

dl2ΓDi,j,Rk,l=m,n((ΓDi,j-Rk,lm,n)2

Each distortion is stored and the minimum is chosen. The transformed domain block which is found to be the best approximation for the current range block is assigned to that range block, i.e. the coordinates of the domain block along with its a and to are saved into the file describing the transformation. This is what is called the Fractal Code Book.

Γ(Di,j)best=Rk,l

Decoding

The reconstruction process of the original image consists on the applications of the transformations describe in the fractal code book iteratively to some initial image Winit, until the encoded image is retrieved back. The transformation over the whole initial image can be described as follows:

Ω1=ηΩinit

Ω2=ηΩ2

Ω3=ηΩ3

…..=………..

Ω4=η(Ωn-1)

η(Ω) can be expressed as two distinct transformations:

η=Γ(Ω)Ψ(Ω)

Γ(Ω) represents the down sampling and low-pass filtering of an image ηΩ to create a domain image e.g. reducing a 128x128 image to a 64x64 image as we describe previously. Ψ(Ω) represents the ensemble of the transformations defined by our mappings from the domain blocks in the domain image to the range blocks in the range image as recorded in the fractal. Δn will converge to a good approximation of Δorig in less than 7 iterations.

CHAPTER 4: ITERATED FUNCTION SYSTEM

4.1 Introduction

Let  us  consider  the  case  of  the  photocopying  machine  which  we  saw  earlier. Suppose  we  were  to  feed  the  output  of  this  machine  back  as  input  and  continue  the  process  iteratively. We  can  find  that  all  the  output  images  seem  to  be  converging  back  to  the  same  output  image  and  also  that  the  final  image  is  not changed  by  the  process. We call this image the attractor for the copying machine.

Because  the  copying  machine  reduces  the  input  image  the  copies  of  the  initial  image  will  be  reduced  to  a  point  as  we  repeatedly  feed  the  output  back  as  input; there  will  be  more  and  more  copies  but  the  copies  at  each  stage  get  smaller  and  smaller. So  the  initial  image  doesn’t  affect  the  final  attractor ; in  fact, it  is  only  the  position  and  orientation  of  the  copies  that  determines  what  the  final  image  will  look  like.

The  final  result  is  determined  by  the  way  the  input  image  is  transformed , we  only  describe  these  transformations. Different transformations lead to different attractors  with  the  technical  limitation  that  the  images  must  be  contractive; i.e, a  given  transformation  applied  to  any  point  in  the  input  image  must  bring  them  closer  in  the  copy. This  technical  condition  is  very  natural  since  if  the  points  in  the  copy  were  to  be  spread  out  the  attractor  might  have  to  be  of  infinite  size.

A  common  feature  of  these  attractors  thus  formed  is  that  in  the  position  of  each of  these  images  of  the  original  square  there  is  a  transformed  copy  of  the  whole image. Thus the images thus formed are all fractals. This method of creating fractals was put forward by John Hutchinson.

M.Barnsley  suggested  that  perhaps  storing  images  as  collections  of  transformations  could  lead  to  image  compression. The  complex  figure  of  a  fern  is  generated  from just  four  affine  transforms. Each  affine  transform  is  defined  by  six  numbers ‘a’, ‘b’, ‘c’, ‘d’, ‘e’  and  ‘f’. Storing  these  on  a  computer  do  not  require  much  memory. Suppose  we  are  required  to  store  the  image  of  a  face. If  a  small  number  of  transformations  could  generate  that  face  then  it  could  be  stored  compactly.

If  we  scale  the  transformations  defining  any  image  the  resulting  attractor  will  also  be  scaled. This is the implication of fractal image compression. The  extra  detail  needed  for  decoding  at  larger  sizes  is  generated  automatically  by  the  encoding transforms. However  in  some  cases  the  detail  is  realistic  at  low  magnification  and  this  is  a  useful  feature  of  the  method.

Magnification  of  the  original  shows  pixelation; the  dots  that  make  up  the  image  are  clearly  discernable. This is because of the magnification produced.

Standard  image  compression  methods  can  be  evaluated  using  their  compression  ratios : the  ratio  of  the  memory  required  to  store  an  image  as  a  collection  of  pixels  and  the  memory  require  to  store  a  representation  of  the  image  in  compressed  form.

The  compression  ratio  of  fractal  is  easy  to  misunderstand  since  the  image  can  be decoded  at  any  scale. In  practice  it  is  important  to  either  give  the  initial  and  decompressed  image  sizes  or  use  the  same  sizes  for  a  proper  evaluation. Because  the  decoded  image  is  not  exactly  the  same  as  the  original  such  schemes  are  said  to  be  lossy.

4.2 Mathematical model

A  mathematical  model  for  the  copying  machine  described  earlier  is  called  an  Iterative  Function  System (IFS). An  IFS  system  consists  of  a  collection  of  contractive  transformations  wi , this  collection  defines  a  map, W(s) =iwi(s), where I = 1, 2, ….,n. Where  s  is  the  input  and  W(s)  is  the  output  of  the  copier. Whatever  the  initial  image  S, the  image  after  infinite  interactions  will  tend  to  the  attractor  Xw. This Xw is unique for that particular W(s).

Two important facts are listed as below:

When  the  wi  are  contractive  in  the  plane  then  W  is  contractive  in  a  ste  of  the  plane. This was proved by Hutchinson.

If  we  are  given  a  contractive  map  W  on  a  space  of  images  then  there  is  a  special  image  called  the  attractor  denoted  by  Xw  with  the  following  properties.

- The attractor is called the fixed point of W

- Xw is unique. If  we  find  any  set  S  and  an  image  transformation  w  satisfying  W(s) =S, then  S  is  the  attractor  of  W.

Fractal image representation can be described mathematically as an iterated function system (IFS).

For Binary Images

We begin with the representation of a binary image, where the image may be thought of as a subset of . An IFS is a set of contraction mappings ƒ1,...,ƒN,

According to these mapping functions, the IFS describes a two-dimensional set S as the fixed point of the Hutchinson operator

That is, H is an operator mapping sets to sets, and S is the unique set satisfying H(S) = S. The idea is to construct the IFS such that this set S is the input binary image. The set S can be recovered from the IFS by fixed point iteration: for any nonempty compact initial set A0, the iteration Ak+1 = H(Ak) converges to S.

The set S is self-similar because H(S) = S implies that S is a union of mapped copies of itself:

So we see the IFS is a fractal representation of S.

Extension to Grayscale

IFS representation can be extended to a gray-scale image by considering the image's graph as a subset of . For a gray-scale image u(x,y), consider the set S = {(x,y,u(x,y))}. Then similar to the binary case, S is described by IFS using a set of contraction mappings ƒ1,...,ƒN, but in ,

Encoding

A challenging problem of ongoing research in fractal image representation how to choose the ƒ1,...,ƒN such that its fixed point approximates the input image, and how to do this efficiently. A simple approach for doing so is the following:

- Partition the image domain into blocks Ri of size s×s.

- For each Ri, search the image to find a block Di of size 2s×2s that is very similar to Ri.

- Select the mapping functions such that H(Di) = Ri for each i.

In the second step, it is important to find a similar block so that the IFS accurately represents the input image, so a sufficient number of candidate blocks for Di need to be considered. On the other hand, a large search considering many blocks is computationally costly. This bottleneck of searching for similar blocks is why fractal encoding is much slower than for example DCT and wavelet based image representations.

Features

With fractal compression, encoding is extremely computationally expensive because of the search used to find the self-similarities. Decoding however is quite fast. While this asymmetry has so far made it impractical for real time applications, when video is archived for distribution from disk storage or file downloads fractal compression becomes more competitive.

At common compression ratios, up to about 50:1, Fractal compression provides similar results to DCT-based algorithms such as JPEG.  At high compression ratios fractal compression may offer superior quality. For satellite imagery, ratios of over 170:1 have been achieved with acceptable results. Fractal video compression ratios of 25:1-244:1 have been achieved in reasonable compression times (2.4 to 66 sec/frame).

Compression efficiency increases with higher image complexity and color depth, compared to simple gray-scale images.

4.3 Resolution independence and fractal scaling

An inherent feature of fractal compression is that images become resolution independent after being converted to fractal code. This is because the iterated function systems in the compressed file scale indefinitely. This indefinite scaling property of a fractal is known as "fractal scaling".

4.4 Fractal interpolation

The resolution independence of a fractal-encoded image can be used to increase the display resolution of an image. This process is also known as "fractal interpolation". In fractal interpolation, an image is encoded into fractal codes via fractal compression, and subsequently decompressed at a higher resolution. The result is an up-sampled image in which iterated function systems have been used as the inter-polant. Fractal interpolation maintains geometric detail very well compared to traditional interpolation methods like bilinear interpolation and bi-cubic interpolation.

*****

CHAPTER 5: SELF SIMILARITY IN IMAGES

Natural images are not self-similar. A  typical  image  of  a  face  does  not  contain  the  type  of  self-similarity  found  in  the  fractals. The  image  does  not  appear  to  contain  affine  transformations  of  itself. But this image contains a different type of self-similarity. Here  the  image  is  formed  of  properly  transformed  parts  of  itself. These  transformed  parts  do  not  fit  together  in  general  to  form  an  exact  copy  of  the  original  image  and  so  we  must  allow  some  amount  of  error in  our  representation  of  an  image  as  a  set  of  self-transformations. This  means  that  an  image  that  we  encode  as  a  set  of  transformations  will  not  be  an  identical  copy  but  an  approximation.

To  get  a  mathematical model  of  an  image  we  express  it  as  a  function  z = f(x,y) where z  is   the  grayscale.

Now, we want to find a map W which takes an input image and yields an output image. If we want to know when W is contractive, we will have to define a distance between two images. The distance is defined as

δf,g=sup(x,y)∈P|fx,y-gx,y|

where f and g are value of the level of grey of pixel (for gray-scale image), P is the space of the image, and x and y are the coordinates of any pixel. This distance defines position (x,y) where images f and g differ the most.

Natural images are not exactly self-similar. Lena image, a typical image of a face, does not contain the type of self-similarity that can be found in the Sierpinski triangle. But next image shows that we can find self-similar portions of the image.

| Original image | Self-similar portions of the image |
| --- | --- |

A part of her hat is similar to a portion of the reflection of the hat in the mirror. The main distinction between the kind of self-similarity found in the Sierpinski triangle and Lena image is that the triangle is formed of copies of its whole self under appropriate affine transformation while the Lena image will be formed of copies of properly transformed parts of itself. These parts are not exact the same; this means that the image we encode as a set of transformations will not be an identical copy of the original image. Experimental results suggest that most images such as images of trees, faces, houses, clouds etc. have similar portions within itself.

5.1 Metric on Images

If  we  want  to  know  whether  W  transformation  is  contractive  we  will  have  to  define  a  distance  between  two  images. A  metric  is  a  function  that  measures  the  distance  between  two  images. There  are  metrics  that  measure  the  distance  between  two  images , the  distance  between  two  points , distance  between  two  sets etc.

Two of the most commonly used metrics are:

Dsup(f,g) = Sup(f(x,y) – g(x,y) )

And the rms metric

Drms(f.g) = S(f(x,y) – g(x,y) )2

The  supremium  metric  finds  the  position  where  two  images  f  and  g  differ  the  most  and  sets  this  value  as  the  distance  between  the  two  points. The rms metric is more convenient  in applications.

5.2 Partitioned IFS

The  individual  transformations  described  earlier  are  performed  on  parts  of  the  image  and  not  on  the  whole. This forms a partitioned IFS also called a PIFS. There are  two  spatial  dimensions  and  the  grey  level  adds  a  third  dimension. A  portion  of  the  original  image  called  domain (Di)  is  mapped  to  a  part  of  the  produced  image  called  range (Ri)  by  the  transformation  Wi. Since  W(f)  is  an  image  the  Ri  covers  the  whole  square  page  and  are  adjacent  but  not  overlapping.

In  the  PIFS  case, a  fixed  point  or  attractor  is  an  image  f  that  satisfies  W(f) =f. Thus  if  we  apply  the  transformations  to  the  image  we  get  back  the  original image. Thus  if  we  are  required  to  code  a  natural  image this  is  first  partitioned  to  several  domain  blocks  and  iterative  function  systems  are  formed  of  the  different  parts  of  the  image  to  yield  the  output  image.

5.3 Partitioning of Images

The  basic  idea  behind  partitioning  of  images  is  to  first  partition  the  image  by  some  collection  of  ranges  Ri, then  for  each  Ri  seek  from  some  collection  of  image  pieces  a  Di  that  has  a  low  rms  error  when  mapped  to  Ri. If  we  know  Ri  and  Di  then  we  can  determine  the  remaining  co-efficient. The various partitioning schemes used are dealt in the next chapter.

*****

CHAPTER 6: ALGORITHM & CODE

6.1 Algorithm for Fractal Image Compression

- Input a binary image, call it M.

- Cover M with square range blocks. The total set of range blocks must cover M, without overlapping.

- Introduce the domain blocks D; they must intersect with M. The sides of the domain blocks are twice the sides of the range blocks.

- Define a collection of local contractive affine transformations mapping domain block D to the range block Ri.

- For each range block, choose a corresponding domain block and symmetry so that the domain block looks most like the part of the image in the range block.

- Write out the compressed data in the form of a local IFS code.

- Apply a lossless data compression algorithm to obtain a compressed IFS code.

In practice these steps can be carried out on a digital image.  The compression is attained by storing the coefficients of the transformations, rather than storing the image pixel by pixel.  The following is an explanation of 'fenc', a simple fractal image compression program and 'fdec' the corresponding decompression program, both included after the explanation.

These two programs can be run on MATLAB and only compress grayscale square images that are in ‘tif’ format, although further changes can be implemented later to account for non-square images and other formats.  The transformation coefficients are saved as mat files are saved representing the compressed images. 'fenc' searches for the transformation with least error from domain blocks to range blocks.  So, the program searches for a transformation until it finds a transformation with dist < dmin, where the expression of dmin is given below.

First, the user must enter the name of the tif image file in the first line of the program: M = imread ('lena_gray_256.tif'). Then, the user specifies the desired range block size by setting rsize equal to the length of the side of the desired range block.  Presently, rsize is set equal to 4, which allows range blocks of size .  We next create the domain blocks, which are twice the size of the range blocks, in this case .  In determining which mapping will need to be made from the domain blocks to the range blocks, we will need to compare the domain blocks to the range blocks.  To accurately compare these blocks, they must be the same size. So, we do some averaging over the domain blocks which allows us to shrink the domain blocks to half of its size in order to match the size of the range blocks.

Originally, each domain block is 8x8.  The averaging only takes place over each distinct block of 2x2 pixels within the domain block. Then the average gray-scale value in each 2x2 block of pixels is represented in one pixel in the scaled domain blocks, called M1.  M1 is a 4x4 block at this point. We subtract the average of the domain block from each entry in the domain block to account for possible darkening of the decompressed image.  The resulting scaled domain block is D.

Now, we save 8 different transformations of each domain block in an eight dimensional monster matrix called bigM.  The transformations include the original domain block, 900, 1800 and 2700 rotation, a horizontal flip, and a vertical flip, as well as the transform of the domain block and a 1800 rotation of the transformed domain block.  We introduce a vector s, which contains different specific scalings to transform the grayscale of the domain block to make a better match to a range block.

At this point, 'fenc' goes through all of the range blocks and offsets each of them by subtracting the average of the range block from each entry in the range block.  Now we can equally compare the domain to the range blocks.  We save the offset of the range blocks in o, which we will add back to the image later.

Next the program cycles through each domain block Di,j and tests each symmetry that is stored in bigM, along with the four possible gray scales for the best transformation that will map to a given range block Rk,l .  When the best map is found, the location of that domain block i0 and j0, the best symmetry m0 of the domain block, the best scaling s0, and the difference of mean of Range and corresponding Domain g0 is saved in the five dimensional matrix T(k,l,:)=[i0 j0 m0 s0 g0].  It is the entries of this matrix that determine the number of bytes needed to store the compressed image file.  The program saves the number of rows of the original image, the size of the range blocks, and the time the program took to achieve the compression.  The time is recorded to compare the results with the amount of time the process took. Once this information is saved in a file, it is possible to compress that file even more by applying a lossless coding algorithm.  It is from the matrix, T, that the program 'fdec' can regenerate the image.

It is important to note that each transformation from the original 8x8 domain is a contraction mapping because the domain must be scaled by ½ in order to map the domain to the range.  Also, the information stored in each (k,l,:) entry of T represent the coefficients of the mappings wi , i = 1,2,3,…N  that make up the N local IFS mappings. The image regenerated after all the mappings in T are applied to some seed image, is the attractor of the local IFS.

In order to regenerate the attractor of the contractive transformations found, we must use the program 'fdec' along with the saved information from 'fenc'.  First we load the correct data using the name that we saved it under in the ‘.mat’ file.  Then we initialize a matrix to perform the mappings on.  This matrix must be the same size as the original image.  As we discussed with IFS, it makes no difference what seed image is used. Although, in the program we initialize the seed image to all ones, which is a uniformly gray image, choosing another image as the seed to the local IFS will arrive at the same result.

Depending on the block size chosen for the range blocks, one may need to vary the number of iterations applied to the seed image in order to arrive at the attractor image.  As more iteration of the IFS is applied to the image, the clearer the attractor will become. After the nth iteration, the image produced corresponds to the An compact set as discussed in the local IFS theory.  First, the domain blocks of the seed image must be created and rescaled to the size of the range blocks.  Then using the T matrix, the domain blocks are transformed and mapped to the range blocks.  This process is repeated for each iteration.  The attractor, M, is then output to be displayed on the screen. The quality of the attractors vary depending on the size of the range blocks used and the error allowed in finding an appropriate transformation form domain block to range block.

Our implementation of this simple method of fractal compression produced great compression ratios.  Considering that each pixel requires 8 bits to store the values of 0 to 255, to store an 256x256 image pixel by pixel would require 65536 bytes (around 65KB).  Using 'fenc' and 'fdec', to store an image of this size with a range block size of  pixels only requires 11776 bytes.  The compression ratio is better than 5:1. Of course, increasing the range block size to  pixels improves the compression to only 2688 bytes, with a compression ratio of approximately 24:1.  The larger range block sizes allow higher compression ratios.  The use of the image will determine the required amount of compression and image quality.

6.2 The MATLAB® code for fractal compression [fenc.m]:

clc;

clf;

% Set timers

begrun=clock

cpu=cputime

M=imread('lena_gray_256.tif');

[sv sh]=size(M);

if sv~=sh

display('Matrix is not square');

return

end

% Begin batch runs

min0=100;

rsize=4;

nd=sv/rsize/2;

nr=sv/rsize;

% Scale the Domain Blocks

for i=1:rsize*nd

for j=1:rsize*nd

M1(i,j)=mean(mean(M((i-1)*2+1:i*2,(j-1)*2+1:j*2)));

end

end

% Matrix of 4 possible scalings to transform grayscale

s=[0.45 0.60 0.80 1];

% Create monster matrix containing all possible 2D transformations

% of the domain blocks. Store in multidimensional matrix bigM.

for i=1:nd

i1=(i-1)*rsize+1;

i2=i*rsize;

for j=1:nd

j1=(j-1)*rsize+1;

j2=j*rsize;

D=M1(i1:i2,j1:j2);

bigM(i1:i2,j1:j2,1)=D;

tmp=rotmat(D);

bigM(i1:i2,j1:j2,2)=tmp;

tmp=rotmat(tmp);

bigM(i1:i2,j1:j2,3)=tmp;

tmp=rotmat(tmp);

bigM(i1:i2,j1:j2,4)=tmp;

bigM(i1:i2,j1:j2,5)=fliph(D);

bigM(i1:i2,j1:j2,6)=flipv(D);

bigM(i1:i2,j1:j2,7)=D';

bigM(i1:i2,j1:j2,8)=rotmat(rotmat(D'));

end

end

% Compare the range blocks and scaled domain blocks.

% k,l - used to cycle through blocks Rkl.

clear T;

for k=1:nr

k1=(k-1)*rsize+1;

k2=k*rsize;

for l=1:nr

l1=(l-1)*rsize+1;

l2=l*rsize;

R=M(k1:k2,l1:l2);

% Offset o is the average in the block Rkl

o=mean(mean(R));

R=double(R);

% Initialize error to large value

dmin=10^9;

i0=0;

j0=0;

m0=0;

% Now cycle through each Domain Dij

for i=1:nd

i1=(i-1)*rsize+1;

i2=i*rsize;

for j=1:nd

j1=(j-1)*rsize+1;

j2=j*rsize;

% Test each transformation

for n=1:4

for m=1:8

D=s(n)*bigM(i1:i2,j1:j2,m);

del_g=o-mean(mean(D));

D=D+del_g;

sum_dist=sum(sum((R-D).^2));

dist=sqrt(sum_dist);

if dist<dmin

dmin=dist;

i0=i;

j0=j;

m0=m;

s0=s(n);

g0=del_g;

end

end

end

end

end

T(k,l,:)=[i0 j0 m0 s0 g0];

end

end

% Stop the clock, store computation time in tim

% and elapsed cpu time in cpu0.

cpu0=cputime-cpu

stoprun=clock

tim=etime(begrun,stoprun)

% Save data in mat file - need to change the name after each use.

save 'gs_norm' sv rsize T tim cpu0;

6.3 The MATLAB® code for fractal image decoding [fdec.m]

load 'gs_norm'

% Initialize matrix

M=100*ones(sv);

% Start Iteration

for iter=1:10

% Enter range block size used in fcomp

rsize=4;

nd=sv/rsize/2;

nr=sv/rsize;

% Rescale Domain Blocks

for i=1:rsize*nd

for j=1:rsize*nd

M1(i,j)=mean(mean(M((i-1)*2+1:i*2,(j-1)*2+1:j*2)));

end

end

% Transform Domain Block Using T matrix

for k=1:nr

k1=(k-1)*rsize+1;

k2=k*rsize;

for l=1:nr

l1=(l-1)*rsize+1;

l2=l*rsize;

i0 = T(k,l,1);

j0 = T(k,l,2);

m0 = T(k,l,3);

s0 = T(k,l,4);

g0 = T(k,l,5);

i1 = (i0-1)*rsize+1;

i2 = i0*rsize;

j1 = (j0-1)*rsize+1;

j2 = j0*rsize;

D = M1(i1:i2,j1:j2);

if m0==2

D=rotmat(D);

elseif m0==3

D=rotmat(rotmat(D));

elseif m0==4

D=rotmat(rotmat(rotmat(D)));

elseif m0==5

D=fliph(D);

elseif m0==6

D=flipv(D);

elseif m0==7

D=D';

elseif m0==8

D=rotmat(rotmat(D'));

end

R=s0*D+g0*ones(size(D));

MM(k1:k2,l1:l2)=R;

end

end

M=MM;

end

% Output Image which is in M

imagesc(M)

colormap(gray);

A Fast Variance-Ordered Domain Block Search Algorithm for Fractal Encoding

CHAPTER 7: INTRODUCTION

Since Jacquin introduced a practical fractal image coding scheme based on the block-wise iterated function system (IFS), many efforts to reduce the computational encoding complexity have been attempted. The major part of the encoding process which need a long encoding time is known to be the searching step to find the best matched domain block to each range block in order to construct the fractal code.

The range block approximation from a domain block by affine transformation in Jacquin's algorithm can be written as

Rk=Im(αn.Dk+∆g)

where Im , αn and ∆g are the isometry transformation, gray level scale factor and luminance shift factor, respectively, and Dk is a spatially contracted domain block. The luminance shift ∆g is computed so that the average gray levels of the range block and the scaled domain block arc the same. That is,

∆g=ERk-αn.E(Dk

The best estimate, Rk , can be obtained by minimizing the squared Euclidean distance (SED) defined as

dE2Rk,Rk=0≤i,j<BRki,j-Rki,j2

where the block size of the range block is assumed to be BxB pixels.

Finding the best estimate Rk usually requires the exhaustive sequential search of all domain blocks, which would take too long time for practical applications in spite of a high reconstructed image quality.

This paper presents a new fast search algorithm using the squared variance distance (SVD) measure which safely rejects a great number of domain blocks during the search process while maintaining the quality of thc decoded image exactly the same to that of the full search.

7.1 Proposed Algorithm

In searching for the closest domain block to a given range block in the SED sense, we used its correlation with the squared variance distance (SVD), defined as

dV2Rk,Rk=VRk-VRk2

VRk2=0≤i,j<BRki,j-E(Rk)2

ERk=1B20≤i,j<BRki,j

An important observation in real image is that the greater the variance differences between two image-blocks are, the greater the SED becomes. This motivated the use of variance as a feature of the image-block when comparing image-blocks in the SED sense. That is to say, the similarity in the complexity of image blocks, SVD, is a nearly sufficient statistic for the small Euclidean distance, SED, of interblocks. The explicit relationships between SVD and SED for any image blocks can be given by the inequality

dE2≥dV2

as shown in Appendix. Therefore, if the search for the minimum SED domain block starts with the minimum SVD domain block and proceeds to the next nearer ones in the SVD since, there is a high probability of arriving at the minimum SED domain block in only a few searching steps.

Let dmin2 be a known current minimum Euclidean distance of Rk represented by a certain domain block. For any domain block Dk, so for Rk, if the inequality

VRk-VRk2≥dmin2

is satisfied, then Dk will not be the closest domain block to Rk  and it is unnecessary to calculate dV2Rk,Rk. This fact facilitated the early termination of our search process without worrying over the possible existence of closer domain blocks in the remaining domain pool. For our purposes, all of the domain blocks in the domain pool are sorted in descending (or ascending) order of their variances.

N contrast scale factors (αn) are applied to find the closest domain block in fractal encoding as shown in Eq. (1).

For the contrast scaled domain block, the variance can be easily calculated by the following equation:

VDkαn2=αn2×VDk2

Therefore, our partial domain block search algorithm can be adopted to find the closest contrast scaled domain block in the same manner without a great deal of additional computational complexity. The composition of the domain pool and the search procedure for our algorithm are illustrated in Fig. 1.

All of the domain blocks are sorted according to their variances in descending order, assuming that the original image is partitioned into overlapping M domain blocks and 4 contrast scalings α1=1, α1>α2>α3>α4 are applied. Including the contrast scaled domain blocks whose variances can be obtained from Eq. (9), 4M domain blocks are produced totally and sorted in the domain pool.

For a range block Rk whose variance is VR, the search process starts from the domain block having the closest variance to VR, i.e. the uth domain block Du, in Fig. 1. With the minimum SVD domain block Du, regarded as the reference block, the upper neighboring domain block Du-1 is searched, then the lower neighboring domain block Du+1 is searched afterwards. This search process continues in a back-and-forth manner until the termination condition, Eq. (8), is satisfied in both directions, at u-2th  and u+3th  domain block in this case. The rest of the lower index domain blocks than the u-2th  domain block and the u-2th  domain block are rejected as a whole because their SVD are so large that there is no possibility of being closer to the range block than the domain block having current minimum SED. The rest of the higher-index domain blocks than the u+3th  domain block and the u+3th domain block are rejected as a whole for the same reason.

7.2 The MATLAB® code of variance-ordered partial search (VPS) algorithm:

clc;

clf;

% Set timers

begrun=clock

cpu=cputime

M=imread('lena_gray_256.tif');

[sv sh]=size(M);

if sv~=sh

display('Matrix is not square');

return

end

rsize=4;

nd=sv/rsize/2;

nr=sv/rsize;

% Scale the Domain Blocks

for i=1:rsize*nd

for j=1:rsize*nd

M1(i,j)=mean(mean(M((i-1)*2+1:i*2,(j-1)*2+1:j*2)));

end

end

% Matrix of 4 possible scalings to transform grayscale

s=[0.45 0.60 0.80 1];

% Find variances of domain block for different scalings

Vd=zeros(nd,nd,4);

for i=1:nd

i1=(i-1)*rsize+1;

i2=i*rsize;

for j=1:nd

j1=(j-1)*rsize+1;

j2=j*rsize;

D=s(n)*M1(i1:i2,j1:j2);

D=double(D);

Z=mean(mean(D));

Z=D-Z;

for n=1:4

Vd(i,j,n)=(s(n)^2)*sum(sum(Z.^2));

end

end

end

% Sorting variances of domain block

Vd_ord=zeros(4,nd*nd*4);

Vd_ord=Vd_ord';

w=Vd(:);

Vd_ord(:,1)=w;

e=1:nd;

e=e';

e=e*ones(1,nd);

e=e(:);

e=e*ones(1,4);

e=e(:);

f=1:nd;

f=f'*ones(1,nd);

f=f';

f=f(:);

f=f*ones(1,4);

f=f(:);

g=[1 2 3 4];

g=g';

g=g*ones(1,nd*nd);

g=g';

g=g(:);

Vd_ord(:,2)=e;

Vd_ord(:,3)=f;

Vd_ord(:,4)=g;

for k=1:(nd*nd)

for i=1:(nd*nd-1)

temp=zeros(1,4);

if Vd_ord(i,1)<Vd_ord(i+1,1)

for j=1:4

temp(j)=Vd_ord(i,j);

Vd_ord(i,j)=Vd_ord(i+1,j);

Vd_ord(i+1,j)=temp(j);

end

end

end

end

% Begin batch runs

clear T;

% Compare the range blocks and scaled domain blocks.

% k,l - used to cycle through blocks Rkl.

for k=1:nr

k1=(k-1)*rsize+1;

k2=k*rsize;

for l=1:nr

l1=(l-1)*rsize+1;

l2=l*rsize;

R=M(k1:k2,l1:l2);

R=double(R);

% Offset o is the average in the block Rkl

O=mean(mean(R));

Rm=R-O;

Vr=sum(sum(Rm.^2));

min=10000;

u=1;

for i4=1:(nd*nd)

er=(sqrt(Vr)-sqrt(Vd_ord(i4,1)))^2;

if er<min

min=sqrt(er);

u=i4;

end

end

% Initialize error to large value

i0=0;

j0=0;

m0=0;

g0=0;

s0=0;

dmin=10^9;

stf=0;

q=zeros(1,2);

q(1)=u-1;

q(2)=u;

while stf==0

dir=1;

while (dir>=0)&(stf==0)

if ((sqrt(Vd_ord(q(dir+1),1))-sqrt(Vr))^2)>=dmin^2

stf=2;

else

i=Vd_ord(q(dir+1),2);

j=Vd_ord(q(dir+1),3);

n=Vd_ord(q(dir+1),4);

% Now cycle through each Domain Dij

i1=(i-1)*rsize+1;

i2=i*rsize;

j1=(j-1)*rsize+1;

j2=j*rsize;

bigM=zeros(rsize,rsize,8);

D=M1(i1:i2,j1:j2);

del_g=O-s(n)*mean(mean(D));

D=D+del_g;

bigM(:,:,1)=D;

tmp=rotmat(D);

bigM(:,:,2)=tmp;

tmp=rotmat(tmp);

bigM(:,:,3)=tmp;

tmp=rotmat(tmp);

bigM(:,:,4)=tmp;

bigM(:,:,5)=fliph(D);

bigM(:,:,6)=flipv(D);

bigM(:,:,7)=D';

bigM(:,:,8)=rotmat(rotmat(D'));

% Test each transformation

for m=1:8

D=bigM(:,:,m);

sum_dist=sum(sum((R-D).^2));

dist=sqrt(sum_dist);

if dist<dmin

dmin=dist;

i0=i;

j0=j;

m0=m;

s0=s(n);

g0=del_g;

end

end

end

q(dir+1)=q(dir+1)+((-1)^dir);

if ((q(dir+1)<1)|(q(dir+1)>(nd*nd*4)))==1

stf=2;

end

end

end

T(k,l,:)=[i0 j0 m0 s0 g0];

end

end

% Stop the clock, store computation time in tim

% and elapsed cpu time in cpu0.

cpu0=cputime-cpu

stoprun=clock

tim=etime(begrun,stoprun)

% Save data in mat file - need to change the name after each use.

save 'gs_ffic' sv rsize T tim cpu0;

The transformation co-efficeints were saved onto a file named “gs_ffic” MATLAB data file (‘.mat’).

The fractal compression decoder file is same as that of the normal fractal compression, without any changes.

*****

CHAPTER 7: FRACTAL IMAGE COMPRESSION VERSUS JPEG COMPRESSION

At high compression ratios, fractal compression has similar compression and quality performance as the JPEG standard. Let see some comparisons:

Original  image (184,320 bytes)

| JPEG-max. quality (32,072) comp. ratio: 5.75:1 | FIF-max. quality (30,368) comp. ratio: 6.07:1 |
| --- | --- |

| JPEG-med. quality (11,278) comp. ratio: 16.34:1 | FIF-med. quality (7,339) comp. ratio: 25.11:1 |
| --- | --- |

| JPEG-min. quality (8,247) comp. ratio: 22.35:1 | FIF-min. quality (3,924) comp. ratio: 46.97:1 |
| --- | --- |

These are expected results for Fractal Compression. Some images can be compressed over 80:1 or higher. We can see worse image quality for higher compression ratios.

Fractal Compression is an asymmetric process, compression taking a long time compared with decompression. JPEG/DCT is symmetric process, encoding and decoding taking the same amount of time. Fractal compression times can be decreased by using a dedicated hardware. For example, for a 640 x 400 pixel 24 bit colour image, JPEG software compression and decompression took 6 seconds each, for fractal compression the time was 68 seconds and decompression was only one second. Reading the original uncompressed image required two seconds.
Another one very important feature of the Fractal Image Compression is Resolution Independence.

Fractal encoding image has no natural size. It is set of transformations defined on domains and ranges of the image. When we want to decode image, only thing we have to do is apply these transformations on any initial image (it is usually grey rectangle for grey-scale images) iteratively. Every iteration is step closer to the decoded image. After each iteration, details on the decoded image are sharper and sharper. That means, the decoded image can be decoded at any size. The extra detail needed for decoding at larger sizes is generated automatically by the encoding transforms. The next question is: if we decode an image at larger and larger sizes, will we be able to see atoms of the Lena's face? The answer is, of course, no because we are encoding digitized image which is only approximation of the nature image but on the larger sizes we do not have "pixelization" effect than we get sharper details.

| Lena's eye original image enlarged to 4 times | Lena's eye decoded at 4 times its encoding size |
| --- | --- |

This feature of Fractal Image Compression is unique. The greatest irony of the coding community is that great pains are taken to precisely measure and quantify the error present in a compressed image, and great effort is expended toward minimizing an error measure that most often is - let us be gentle - of dubious value. These measures include signal-to-noise ratio, root mean square error, and (less often) mean absolute error. A simple counter example is systematic error: add a value of 10 to every pixel. Standard error measures indicate a large distortion, but the image has merely been brightened.

With respect to those dubious error measures, the results of tests reveal the following: for low compression ratios JPEG is better, for high compression ratios fractal encoding is better. The crossover point varies but is often around 40:1 to 50:1. This figure bodes well for JPEG since beyond the crossover point images so severely distorted that they are seldom worth using.

Proponents of fractal compression counter that signal-to-noise is not a good error measure and that the distortions present are much more 'natural looking' than the “block”y effect of JPEG, at both low and high bit rates. This is a valid point but is by no means universally accepted.

*****

CHAPTER 9: APPLICATIONS OF FRACTALS

Creating a fractal requires just a few lines of software. The  resulting  picture  could  be  quite  rich  in  details and  would  require  large  memory  if  stored  as  such. This forms the basis of fractal compression of pictures. Given  any  arbitrary  picture  one  has  to  find  out  which  of  the  portions  of  images  could  be  thought  of  as  self-similar  versions  of  other  portions. Self-affine transformations can then be found out. The  image  can  then  be  displayed  quickly  and  at  any  magnification  with  infinite  levels  of  fractal  detail. Genetic  algorithms  in  MATLAB  are  generally  used  for  the  efficient  encoding  of  fractals.

The most amazing thing about fractals is the variety of their applications. Besides theory, they were used to compress data in the Encarta Encyclopaedia and to create realistic landscapes in several movies like Star Trek.

The places where you can find fractals include almost every part of the universe, from bacteria cultures to galaxies to your body. In this section, we have the most important applications, trying to include them from as many areas of science and everyday life as possible. Most applications use a specific topic related to fractals that is discussed either in the tutorial or types of fractals. Examples are  Galaxies, Rings of Saturn, Bacteria Cultures, Chemical Reactions, Human Anatomy, Molecules, Plants , Population Growth, Clouds, Coastlines and Borderlines, Data Compression, Diffusion, Economy, Fractal Art, Fractal Music, Landscapes, Newton's Method, Special Effects (Star Trek), Weather.

A fractal created using the program                           High voltage breakdown within a 4″ block

Apophasis and a julian transform                              of acrylic creates a fractal Lichtenberg figure

Fractal branching occurs in a fractured                       A fractal is formed when pulling apart

surface such as a microwave-irradiated DVD.                                  two glue-covered acrylic sheets

So why isn't everyone using Fractal Image Compression?

Fractal Image Compression is still under development. Many different researchers and companies are trying to develop new algorithms to reach shorter encoding time and smaller files. But there are still some problems with fractal compression. Fractal Compression Standard does not exist ! FIF, Fractal Image Format is not standardized, is not embedded into any browsers .In spite of all, total number of publications on fractal image compression is growing; over 280 papers was published last year.

*****

CHAPTER 10: RESULTS

We encoded Lena (256x256) grey-scale image using the MATLAB® program code described above. This is performed using the 4x4 block size using the normal fractal compression method and the fast fractal compression method. Here is a summary of the results:

|  | Normal Fractal Compression | Fast Fractal Compression |
| --- | --- | --- |
| Block Size | 4x4 | 4x4 |
| Time to encode | 7.6798×103secs | 253 secs |
| Time to decode | 32 secs | 28 secs |
| Size of fractal code | 30.3 KB | 23.7 KB |

* PC used: Intel Core-i7, Clock speed 3.1 Ghz, 8GB RAM, NVIDIA GEFORCE GT 2 GB Graphics, 1 TB Hard Disk

Original image ‘Lena’ size 256x256

Lena image after fractal decoding with block size 4x4

Lena image after decoding from fast fractal compression with block size 4x4

CHAPTER 10: CONCLUSIONS

In this paper, we proposed a new fast fractal encoding algorithm using the variances of image blocks. The experimental results showed that the numbers of searched domain blocks are reduced to less than a quarter of that of the full search while maintaining the quality of the decoded image exactly the same.

If the image block classifier is employed in a fractal encoder, the VPS algorithm can also be independently applied to each class such as midrange blocks and edge blocks to reduce the encoding time. Furthermore, since the domain pool is already sorted by the variance of each domain block in the VPS algorithm, a variance block classifier can be effectively adopted to accelerate the fractal image encoding.

REFERENCES:

- Arnaud E. Jacquin, Image Coding Based on a Fractal theory of Iterated Contractive Image Transformations," IEEE Trans. on Image Processing, vol. 1, no. 1, pp. 18-30, Jan. 1992.

- Yuval Fisher, Fractal Image Compression - Theory and Application, Springer-Verlag, ISBN 0-387-94211-4, 1994.

- Yuval Fisher," Fractal Image Compression" ,Course Notes, vol. 12, ACM SIGGRAPH, 1992.

- Dietmar Saupe, \Breaking the Time Complexity of Fractal Image Compression," Technical Report, vol. 53, Institute for Informatics, University at Freiburg, 1994.

- Michael F. Barnsley and Lyman P. Hurd, Fractal Image Compression, AK Peters Ltd.,ISBN 1-56881-000-8, 1993.

- Benoit B. Mandelbrot, The Fractal Geometry of Nature, W.H. Freeman and Company, ISBN 0-7167-1186-9, 1983.

- Maaruf Ali and Trevor G. Clarkson, Fractal Image Compression," Proc. 1st Seminar on Information Technology and it Applications (ITA '91), Sept. 29, 1991,  Leicester.

- www.wikipedia.com